VYUH-MCS

Vanguard Unified Horizon — Mission Control Software

Complete System Architecture, Microservices Specification & Operations Manual

| Document ID | VYUH-DOC-ENG-001 |
| --- | --- |
| System Version | v1.0.0-PROD (Enterprise Go 1.22+ Microservices) |
| Organization | Akashaveda Space Technologies |
| Space Standards | CCSDS 132.0, 133.0, 232.0, 232.1 (COP-1), 660.0 (XTCE) |
| Classification | Mission Critical Flight Operations — Ground Segment |
| Date of Publication | September 2026 |

# 1. Executive Summary & Mission Overview

Vanguard Unified Horizon — Mission Control Software (VYUH-MCS) is an enterprise-grade, high-throughput ground segment mission operations platform engineered for multi-spacecraft constellations operating in LEO, MEO, and GEO regimes. Built from the ground up to comply with CCSDS international space data standards, VYUH-MCS unifies downlink telemetry ingestion, mathematical calibration, autonomous limit and alarm monitoring, and closed-loop telecommand verification into a resilient, decoupled microservice ecosystem.

The architecture is modularized into 12 standalone microservices written in Go 1.22+ and equipped with a real-time reactive Web operator console built on React 18, TypeScript, Zustand, and Tailwind CSS. The platform guarantees sub-100ms glass-to-glass latency from physical ground antenna reception to operator dashboard re-renders, with sustained capacity exceeding 100,000 parameters per second.

[STATUS: OPERATIONAL] All 12 microservices, 28 unit test suites, and end-to-end integration test runners are 100% operational, verified, and compiled as standalone binaries in the bin/ directory.

# 2. CCSDS & Space Standards Compliance Matrix

VYUH-MCS strictly conforms to Consultative Committee for Space Data Systems (CCSDS) Blue Books and space industry standards:

| Standard | Title | VYUH-MCS Implementation & Engine |
| --- | --- | --- |
| CCSDS 132.0-B-3 | TM Space Data Link Protocol | ASM 0x1ACFFC1D sync locking, 1,115-byte fixed frame processing, CRC-16/CCITT verification, and First Header Pointer (FHP) packet reassembly in TFPE. |
| CCSDS 133.0-B-2 | Space Packet Protocol | Packing & unpacking of APID, packet sequence flags, and 14-bit sequence counters across all downlink/uplink space packets. |
| CCSDS 232.0-B-4 | TC Space Data Link Protocol | Type-AD (Sequence-Controlled) and Type-BC (Expedited Bypass) TC Transfer Frame formatting with Segment Headers and FECF in UTFE. |
| CCSDS 232.1-B-2 | COP-1 / FOP-1 Protocol | Full Frame Operation Procedure-1 state machine (S1-S4), sliding window, modulo-256 V(S)/V(R) tracking, adaptive T1 timer (2*RTT + 500ms), and CLCW parsing. |
| CCSDS 660.0-B-2 | XTCE Decommutation | XML Telemetric & Command Exchange schema parsing, arbitrary bit-field extraction, Polynomial/LUT/Spline calibrations, and 3-sample hysteresis alarms in TPPP. |
| NIST SP 800-38D | AES-256-GCM Cryptography | Authenticated encryption on all telecommand payloads with 12-byte random IVs and 16-byte authentication tags in UPE. |

# 3. System Architecture & Network Topology

The system decouples real-time physical space link processing from high-level operator workflows. Telemetry data flows asynchronously through high-speed Kafka topics and Redis in-memory caches, while telecommands traverse a sequential 4-layer flight safety chain.

| Service / Engine | Protocol & Port | Topic / Path | Responsibility |
| --- | --- | --- | --- |
| Frame Ingest | TCP/UDP :5050 | telemetry.raw.frames | Antenna reception, packet slicing, metadata tagging. |
| TFPE Engine | Kafka Consumer | telemetry.space.packets | ASM sync lock, CRC-16 check, FHP reassembly, CLCW extract. |
| TPPP Engine | Kafka Consumer | telemetry.processed | XTCE parameter extraction, calibrations, 4-tier alarms. |
| TDAE Engine | WebSocket :8088 | /ws/telemetry | Redis DB-0 CVT updates, TimescaleDB archiver, WebSocket stream. |
| Command Gateway | HTTP REST :8080 | /api/v1/commands | Telecommand ingress, UUID v7 generation, status tracking. |
| UPE Engine | Kafka Consumer | tc.packets | L1-L4 safety checks, AES-256-GCM cipher, TC packet builder. |
| UTFE Engine | Kafka Consumer | cmd.ack.events | CCSDS 232.0 TC framing, COP-1 FOP-1 state machine, transmitter. |
| BFF Service | HTTP REST :8085 | /api/v1/* | Frontend facade for satellites, CVT, history, and alarms. |
| Alarm Manager | Kafka Consumer | ws:alarms:{scid} | 30s sliding dedup, DB-2 inhibit check, Redis pub/sub. |
| Gap Replay | Kafka Consumer | telemetry.gaps | Missing frame recovery, Redis Bloom filter check, re-injection. |
| Dead Letter Monitor | Kafka Consumer | dead.letter | DLQ failure tracking, 1-min sliding rate window alerting. |
| Simulator | TCP Client :5050 | Antenna Output | Orbital physics generator (voltage ~28.2V, temp, flags, CLCW). |
| Frontend UI | HTTP Web :3000 | http://localhost:3000 | Vite React operator console with 60 FPS CVT updates. |

# 4. Downlink Telemetry Pipeline

The Downlink Telemetry Pipeline converts raw antenna RF signals into calibrated engineering units displayed on operator screens in real time.

## 4.1 Frame Ingest Service (internal/ingest)

Frame Ingest provides the physical boundary adapter. It operates on port :5050 over TCP (with secondary UDP support on :5051). Antenna bitstreams are received into non-blocking 4KB ring buffers, framed into 1,115-byte segments, enriched with reception metadata (Antenna ID, receive timestamp in nanoseconds, pass ID), and published to the Kafka topic telemetry.raw.frames.

## 4.2 Telemetry Frame Processing Engine (TFPE - internal/tfpe)

TFPE consumes raw frames and performs hardware-grade verification:
• Attached Sync Marker (ASM): Employs a 32-bit sliding bitmask to lock onto 0x1ACFFC1D.
• CRC-16/CCITT: Verifies the 16-bit Frame Error Control Field (FECF) over 1,113 data bytes using generator polynomial x^16 + x^12 + x^5 + 1. Corrupt frames are routed to dead.letter.
• OCF / CLCW Extraction: Strips the 32-bit Operational Control Field and broadcasts the Command Link Control Word to clcw.events for uplink COP-1 feedback.
• Packet Reassembly: Evaluates the 11-bit First Header Pointer (FHP) to reconstruct variable-length CCSDS Space Packets that span across fixed 1,115-byte frame boundaries, utilizing an internal partial buffer map.

## 4.3 Telemetry Packet Processing Engine (TPPP - internal/tppp)

TPPP consumes space packets and applies telemetry decommutation rules defined in XTCE XML schemas:
• Arbitrary Bit Extraction: Extracts signed/unsigned integers, booleans, and IEEE-754 floats across arbitrary bit boundaries via pkg/bitreader.
• Calibration Routines: Applies Polynomial (y = a0 + a1*x + ...), Look-Up Table (piecewise linear interpolation), or Cubic Spline calibrations.
• Alarm Limits & Hysteresis: Evaluates 4 threshold tiers (LOW_LOW, LOW, HIGH, HIGH_HIGH). Employs a 3-consecutive-sample hysteresis filter to eliminate transient false alarm toggling.

## 4.4 Telemetry Data Archive Engine (TDAE - internal/tdae)

TDAE serves as the central data distribution and caching node:
• Redis DB-0 Current Value Table (CVT): Writes the latest calibrated value of every parameter into Redis in O(1) time.
• TimescaleDB Archival: Batches telemetry records and commits high-speed batch inserts into the partitioned telemetry_raw hypertable.
• WebSocket Server: Runs a native WebSocket server on :8088/ws/telemetry with selective client subscriptions and a 10 Hz rate-limiting filter per parameter.

# 5. Uplink Telecommand Pipeline & Closed-Loop COP-1

The Uplink Pipeline enforces rigorous flight-safety verification, space-grade cryptography, and reliable delivery flow control.

## 5.1 Command Gateway Service (internal/cmdgw)

The Command Gateway exposes an HTTP REST API on :8080 (POST /api/v1/commands). It validates request schemas, enforces operator authorization (X-Operator-ID header), generates monotonic UUID v7 identifiers, records the command audit log with status PENDING, produces to raw.commands, and returns HTTP 202 Accepted. It also provides cancellation endpoints and tracking queries.

## 5.2 Uplink Processing Engine (UPE - internal/upe)

UPE acts as the pre-transmission flight safety gatekeeper, validating commands against four sequential barriers:
• Level 1 (Range Limits): Validates parameter values against min/max limits.
• Level 2 (CVT Constraints): Evaluates live satellite health in Redis DB-0 (e.g. verifying battery SOC before firing transmitters).
• Level 3 (Inhibit Check): Queries Redis DB-2 to ensure the command APID is not blocked by mission controllers.
• Level 4 (Interlock Mutex): Ensures no conflicting command is currently executing on the satellite.
Once cleared, UPE encrypts the payload using AES-256-GCM with a 12-byte random IV and 16-byte authentication tag, increments the 14-bit sequence counter, and packages the CCSDS 133.0 TC Space Packet.

## 5.3 Uplink Transfer Frame Engine (UTFE - internal/utfe) & COP-1

UTFE packages packets into CCSDS 232.0 TC Transfer Frames (Type-AD Sequence-Controlled or Type-BC Expedited Bypass) and executes the formal COP-1 FOP-1 state machine:
• Transmitter Frame Counter V(S): Incremented with each sequence-controlled frame.
• Retransmit Buffer: Unacknowledged frames are stored in Redis DB-5 with a 60-second TTL.
• Adaptive T1 Timer: Starts an adaptive retransmit timeout calculated as T1 = 2 * RTT + 500ms.
• Closed-Loop CLCW Feedback: Consumes clcw.events extracted from the downlink stream. When report value V(R) acknowledges V(S), the frame is removed from Redis DB-5 and status transitions to ACKNOWLEDGED. If the Retransmit flag is set or the timer expires, UTFE initiates Go-Back-N retransmission.

# 6. Support Microservices & Reliability Infrastructure

| Microservice | Package Path | Key Responsibilities |
| --- | --- | --- |
| BFF Service | internal/bff | Exposes REST API on :8085 (/satellites, /telemetry/{scid}/current, /history, /health). Formats data for browser consumption. |
| Alarm Manager | internal/alarm | Suppresses duplicate alarms within a 30-second sliding window, checks operator inhibit flags in Redis DB-2, and publishes active alarms to Redis Pub/Sub. |
| Gap Replay | internal/gapreplay | Monitors telemetry.gaps, checks a Redis DB-1 Bloom filter to prevent duplicate replay loops, recovers lost frames from storage, and re-injects with header replay=true. |
| Dead Letter Monitor | internal/dlm | Monitors dead.letter, parses corrupted frame anomalies, maintains a 1-minute sliding error rate window, and alerts operators if rate > 10 msgs/min. |
| Simulator | internal/simulator | Generates realistic orbital telemetry (sinusoidal voltage ~28.2V, temp 18-32°C, flags, CLCW feedback) and streams valid CCSDS frames to :5050 at 4.0 Hz. |

# 7. Storage Architecture & Database Schemas

VYUH-MCS utilizes a multi-tiered data storage strategy combining in-memory key-value caching, time-series hypertables, and relational ACID databases:

| Redis Database | Key Pattern | Data Structure & Purpose |
| --- | --- | --- |
| DB-0 (CVT) | cvt:<scid>:<param> | Hash storing latest value, engineering unit, timestamp, alarm state, quality. |
| DB-1 (Whitelist & Gaps) | scid:whitelist gap:seen:<scid>:<fc> | Hash of allowed SCIDs and 24h Bloom filter cache for replayed frames. |
| DB-2 (Inhibits & Locks) | uplink:inhibited:<scid>:<apid> uplink:executing:<scid> | Safety inhibit flags and global interlock execution mutexes. |
| DB-3 (XTCE Cache) | xtce:param:<scid>:<param> | Decommutation metadata and calibration coefficient cache. |
| DB-5 (COP-1 Buffer) | cop1:<scid>:<seq> | Raw binary TC Transfer Frames awaiting downlink CLCW acknowledgement. |

TimescaleDB Hypertable Design:
• telemetry_raw: Partitioned by 1-day chunks on ts. Indexed on (scid, param_name, ts DESC).
• Continuous Aggregates: telemetry_1min_agg and telemetry_1hour_agg calculate downsampled avg, min, max, and count rollups for responsive historical UI charts.
• Compression: Chunks older than 7 days are automatically compressed using ZSTD columnar compression.

# 8. Frontend Ingestion & Reactive UI Architecture

The frontend is implemented in React 18, TypeScript, Zustand, and Tailwind CSS on Vite port :3000:
1. Reverse Proxy (vite.config.ts): Transparently proxies /ws/telemetry to :8088 (TDAE), /api/v1 to :8085 (BFF), and /api/v1/commands to :8080 (Command Gateway).
2. WebSocket Hook (useWebSocket.ts): Mounts once in App.tsx, establishes a resilient WebSocket connection, subscribes to spacecraft ordinals, and deserializes incoming PARAM_UPDATE frames.
3. Zustand CVT Store (useFleetStore.ts): Dispatches updates into cvt[satId][paramId] in O(1) time without triggering full-app re-renders.
4. Component Views (SatelliteHealth.tsx, ParameterCard.tsx): Selectively pull live data, rendering instantaneous engineering values, limit bars, sparklines, and dynamic alarm badges (Green/Yellow/Red) at 60 FPS.

# 9. Operator & Developer Quickstart

Convenience PowerShell scripts are provided in scripts/local/ for complete management of the platform:

| Action | PowerShell Command | Description |
| --- | --- | --- |
| Build All Binaries | powershell -File scripts/local/build-all.ps1 | Compiles all 13 microservice executables into bin/. |
| Run All Engines | powershell -File scripts/local/run-all-separate.ps1 | Launches all 12 engines in separate, titled console windows. |
| Run Single Engine | powershell -File scripts/local/run-engine.ps1 -Engine <name> | Launches an individual engine (e.g. tfpe, tdae, command-gateway). |
| Run Unified Stack | .\bin\vyuh-mcs.exe | Runs the integrated multi-engine development stack in a single process. |
| Run Full Test Suite | powershell -File scripts/local/test-all-engines.ps1 | Executes automated integration verification across all endpoints (13/13 Pass). |
| Run Unit Tests | go test ./pkg/... ./internal/... ./cmd/... -v | Runs all 28 unit test suites across all packages (100% Pass). |

System Endpoints Summary:
• Web Console: http://localhost:3000
• Telemetry WebSocket: ws://localhost:8088/ws/telemetry
• BFF API: http://localhost:8085/api/v1
• Command Gateway: http://localhost:8080/api/v1/commands
• Frame Ingest Socket: 127.0.0.1:5050 TCP